## Republic Hydrometorological

## Service of Serbia

## Analysis of the hydrological situation in the main river basins on the territory of the Republic of Serbia for the year 2024

## BELGRADE, JANUARY 2026.

THE CONTENT

1.	Introduction    ......................................................................................................................1

2.	Danube River    ...................................................................................................................2

2.1	Bezdan hydrological station    ........................................................................................2

2.2	Smederevo hydrological station   ...................................................................................5

3.	Tisa River    .........................................................................................................................8

3.1	Senta hydrological station ..............................................................................................8

4.	Sava River   .......................................................................................................................10

4.1	Sremska Mitrovica hydrological station   ....................................................................	10

5.	Velika Morava River   ......................................................................................................	13

5.1	Ljubicevski Most hydrological station  ......................................................................	13

6.	Conclusion   ......................................................................................................................	16

List of Figures and Tables

Figure 1:	Graphic illustration of hydrological stations locations

Figure 2:	Comparative hydrographs of extreme (min. and max.), average multiannual and  discharges for the 2024 year of the Danube River at the Bezdan hydrological station.......................................................................................................................2

Figure 3:  	Probabilities of exceedance of the average monthly discharges of the Danube River at the Bezdan hydrological station ...........................................................................3

Figure 4: 	Histograms and double mass curves of average monthly discharge volumes of the Danube River at the Bezdan hydrological station....................................................4

Figure 5:  	Comparative hydrographs of extreme (min. and max.), average multiannual discharges and discharges for the 2024 year of the Danube River at the Smederevo hydrological station..................................................................................................5

Figure 6: 	Probabilities of exceedance of the average monthly discharges of the Danube River at the Smederevo hydrological station......................................................................6

Figure 7:	Histograms and double mass curves of average monthly discharges volumes of the Danube River at the Smederevo hydrological station……...........….................7

Figure 8:	Comparative hydrographs of extreme (min. and max.), average multiannual discharges and discharge for the 2024 year of the Tisa River at the Senta hydrological station..................................................................................................8

Figure 9:	Probabilities of exceedance of the average monthly discharges of the Tisa River at the Senta hydrological station...................................................................................9

Figure 10: 	Histograms and double mass curves of average monthly discharge volumes of the Tisa River at the Senta hydrological station…........................................................10

Figure 11:	Comparative hydrographs of extreme (min. and max.), average multiannual discharges and discharges for the year 2024 of the Sava River at the Sremska Mitrovica hydrological station................................................................................11

Figure 12:	Probabilities of exceedance of the average monthly discharges of the Sava River at the Sremska Mitrovica hydrological station ……….…..........................................12

Figure 13: 	Histograms and double mass curves of average monthly discharges volumes of the Sava River at the Sremska Mitrovica hydrological station......................................13

Figure 14: 	Comparative hydrographs of extreme (min. and max.), average multiannual discharges and discharges for the year 2024 of the Velika Morava River at the Ljubicevski Most hydrological station....................................................................14

Figure 15: 	Probabilities of exceedance of the average monthly discharges of the Velika Morava at the Ljubicevski Most hydrological station…….....................................15

Figure 16:	Histograms and double mass curves of average monthly discharge volumes of the Velika Morava River at the Ljubicevski Most hydrological station........................16

Figure 17:	Comparative hydrographs of the Danube, Tisa, Sava and Velika Morava for the year 2024................................................................................................................17

Table 1:	Average monthly discharge values at the Bezdan hydrological station on the Danube River according to WMO recommendations and their categorization (HydroSOS)…................................................................................................….....3

Table 2:	Average monthly discharge values at the Smederevo hydrological station on the Danube River according to WMO recommendations and their categorization (HydroSOS)…….....................................................................................................6

Table 3:	Average monthly discharge values at the Senta hydrological station on the Tisa River according to WMO recommendations and their categorization (HydroSOS)………………………….....................................................................9

Table 4:	Average monthly discharge values at the Sremska Mitrovica hydrological station on the Sava River according to WMO recommendations and their categorization (HydroSOS)….......................................................................................................11

Table 5:	Average monthly discharge values at the Ljubicevski Most hydrological station on the Velika Morava River according to the WMO recommendations and categorization (HydroSOS)..……….………………………………………...…..14

1.	Introduction

For the analysis of the hydrological situation in 2024, data from hydrological stations on the Danube, Tisa, Sava and Velika Morava rivers were used. The selection was made with the aim of obtaining a comprehensive picture of the hydrological situation on all major rivers in Serbia, which receive the largest amounts of water from different geographical regions (the Alps and Central Europe, the Carpathians and the Balkan Peninsula).

On the territory of the Republic of Serbia, data from hydrological stations Bezdan and Smederevo on the Danube, Senta on the Tisa, Sremska Mitrovica on the Sava, and Ljubičevski Most on the Velika Morava were used for hydrological analysis (Figure 1). Historical data on river discharge for the entire measurement period up to and including 2023 were used for the hydrological analysis.

Figure 1.  Graphic illustration of hydrological stations locations

2.	     Danube River

2.1        Bezdan hydrological station

Recorded discharges ​​for the year 2024, together with corresponding multiannual characteristic (minimum, average and maximum) discharges of the Danube River at the Bezdan hydrological station, are presented in Figure 2.

Figure 2. Comparative hydrographs of extreme (min. and max.), average multiannual and discharges of the Danube River at the Bezdan hydrological station

Based on the Danube discharge data for 2024, it can be observed that the discharges were around the multiannual average. Two waves were recorded in June and September. The maximum discharges were recorded in September, when they exceed the values of the maximum envelopes, and the maximum annual mean daily value (Qmax.ann.aver.daily) was 6,690 m3/s on September 25. The minimum value (Qmin.ann.aver.daily) was recorded on September 8 and amounted to 1,290 m3/s. It is interesting that the maximum discharges values ​​that appeared in September far exceeded the maximum envelope for that month.

In accordance with the program of the World Meteorological Organization (WMO) for the establishment of a system for the standardized display of information on the current state of water resources and forecasting the hydrological situation for the next period (HydroSOS), a statistical analysis of hydrological data was performed. The HydroSOS categorization provides 5 levels of values in relation to intervals of characteristic percentages of occurrence (percentiles): notably high, above normal, normal, below normal and notably low. Below, for each hydrological station analyzed, a categorization (according to the HydroSOS categorization) of average monthly discharges values ​​with the limits of the percentile area specified in the tables for the given year is presented in relation to the historical multiannual average values ​​ending with the previous calendar year for the given month. Table 1 shows the values ​​of average monthly discharges at the Bezdan hydrological station on the Danube River according to the HydroSOS categorization.

Table 1:   Average monthly discharge values at the Bezdan hydrological station on the Danube River according to WMO recommendations and their categorization (HydroSOS)

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qaverage |  | 1990 | 2120 | 2410 | 2760 | 2820 | 2900 | 2660 | 2300 | 1920 | 1670 | 1740 | 1890 |
| Qmin |  | 1010 | 1040 | 1190 | 1330 | 1590 | 1620 | 1190 | 1040 | 1030 | 987 | 901 | 939 |
| Percentiles (%) | 10 | 1370 | 1610 | 1770 | 1960 | 2100 | 1720 | 1320 | 1280 | 1040 | 1180 | 1250 | 1250 |
|  | 33 | 1830 | 2080 | 2310 | 2400 | 2420 | 2200 | 1790 | 1600 | 1430 | 1380 | 1540 | 1530 |
|  | 50 | 2030 | 2340 | 2590 | 2780 | 2570 | 2510 | 2230 | 1830 | 1650 | 1580 | 1800 | 1800 |
|  | 67 | 2280 | 2580 | 3050 | 3050 | 2960 | 2780 | 2560 | 2020 | 1840 | 1870 | 2000 | 1990 |
|  | 90 | 2940 | 3280 | 4110 | 3610 | 4220 | 3810 | 3490 | 2690 | 2380 | 2470 | 2800 | 2770 |
| Qmax |  | 3660 | 4260 | 4250 | 5900 | 5750 | 7570 | 6090 | 5530 | 3930 | 2790 | 3910 | 3960 |
| Qaver.monthly,2024 |  | 3830 | 2990 | 2320 | 2310 | 2330 | 4340 | 2630 | 1730 | 3270 | 3150 | 1630 | 1750 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Average annual discharge of the Danube River at the Bezdan hydrological station in 2024 was Qavg.annual = 2690 m3/s. The average values ​​of monthly discharges for 2024, according to the recommendations of the WMO in the profile of the Bezdan hydrological station on the Danube in March, July, November and December were within the limits of normal values. They were notably high in January, February, June, September and October, and below normal in April, May and August.

An additional statistical analysis of the average monthly discharges was performed, and Figure 3 shows the average monthly discharges for 2024 together with the calculated average monthly discharges of characteristic probabilities of occurrence in the profile of the Bezdan hydrological station on the Danube River.

Figure 3: Probabilities of exceedance of the average monthly discharges of the Danube River at the Bezdan hydrological station

The diagram shows that the average monthly discharges in the profile of the hydrological station Bezdan in 2024 ranged from 5% to 80% of exceedance, except in January and October when they reached values ​​of less than 2% of exceedance.The values ​​of the average monthly discharges for January and October 2024 corresponded to an exceedance of about 1%. It is interesting to note that the average daily discharges recorded from January 1 to January 3, as well as January 10, then from September 20 to October 7 and from October 12 to 18, were also the maximum average daily values ​​in those days since the beginning of flow measurement at this hydrological station.

An analysis of monthly discharges volumes was also performed using the histogram of average monthly discharges and double mass volume curves shown in the diagram in Figure 4.

Figure 4.	Histograms and double mass curves of average monthly discharge volumes of the Danube River at the Bezdan hydrological station

Based on the available data on average monthly discharges for the year 2024 at the Bezdan hydrological station on the Danube River, and according to the histogram (Figure 4), it can be concluded that in January, February, June, September and October discharges were significantly above the average multiannual values, while in March, April, May, July, November and December they were around the average. Only in August this value was slightly below the average. In January, September and October, this value was even twice as high as the average.

Overall, the double mass curve of the average monthly discharges was above average in the first two months of 2024, close to average in March, slightly below average in April and May, and then again above average in June. After the average July and August, in September and October the flows were again significantly above the average, and in November and December they were again around the average.

It is interesting to note that the higher water content was at the beginning of the year, as well as during June, September and October, contributed to the fact that at the end of the year the total annual volume of water was significantly higher than the value of the volume of water corresponding to the multiannual average (volume surplus was about 13100 × 106 m3). Based on the above, when it comes to the the Bezdan hydrological station on Danube River, we can consider 2024 as an above-average water year.

2.2	Smederevo hydrological  station

For the section of the Danube River, downstream from the confluence of the Sava and Tisa, the data for the hydrological station Smederevo were analyzed and are shown on the hydrograph in Figure 5.

Figure 5. Comparative hydrographs of extreme (min. & max.), average and averge multiannual discharges of the Danube River at the Smederevo hydrological station

Analyzing the diagram in Figure 5, it can be seen that the discharge values on the Danube ​​River in the profile of the Smederevo hydrological station in January exceeded the maximum envelope (in the period from January 4 to 6). The maximum annual average daily discharge for the year 2024 was recorded on January 6 and amounted to Qmax.ann.avg.daily = 9,280 m3/s. After that increase in January, discharges go into recession, in March they were around the average and in April and May they were below the average. After a new increase in June, discharges move slightly above the average, only to go into recession again during July and August, and in the first decade of September they approach the envelope of the minimum. At that time, the minimum annual mean daily discharge for the year 2024 was recorded on September 8 and amounted to Qmin.ann.avg.daily = 2,150 m3/s. In the middle of September, discharges increase again, so that in the first decade of October, they again exceed the maximum envelope (October 8 and 9). In November, discharges again go into recession, so that in the third decade of November and December, discharges fluctuate less, staying slightly below the average multiannual values.

Table 2 shows the values ​​of mean monthly water discharges at the Smederevo hydrological station on the Danube River according to the WMO program for establishing a system for the standardized display of information on the current state of water resources and forecasting the hydrological situation for the next period (HydroSOS) for a given year in relation to historical multiannual average values ​​for a given month.

Table 2:	Average monthly discharge values at the Smederevo hydrological station on the Danube River according to WMO recommendations and their categorization (HydroSOS)

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qaverage |  | 5130 | 5470 | 6340 | 7240 | 6670 | 5940 | 4930 | 4030 | 3600 | 3610 | 4330 | 5000 |
| Qmin |  | 2460 | 2480 | 3550 | 3460 | 3580 | 2680 | 2170 | 1820 | 1780 | 1840 | 2170 | 2030 |
| Percentiles (%) | 10 | 4340 | 4610 | 4420 | 3790 | 3290 | 2530 | 2410 | 2270 | 2820 | 3100 | 3090 | 1250 |
|  | 33 | 5360 | 6000 | 5870 | 5030 | 4070 | 3190 | 2920 | 2840 | 3510 | 4180 | 4180 | 1530 |
|  | 50 | 6170 | 7070 | 6500 | 5750 | 4690 | 3870 | 3330 | 3450 | 3890 | 4640 | 4640 | 1800 |
|  | 67 | 6920 | 8050 | 7060 | 6450 | 5470 | 4510 | 3860 | 3950 | 4570 | 5550 | 5430 | 1990 |
|  | 90 | 8680 | 10300 | 9350 | 7840 | 6850 | 5810 | 5090 | 5340 | 6490 | 7160 | 7100 | 2770 |
| Qmax |  | 8690 | 8850 | 9940 | 13100 | 12400 | 13000 | 10800 | 8030 | 7410 | 7290 | 9750 | 9240 |
| Qaver.monthly,2024 |  | 8370 | 6870 | 6120 | 5480 | 4870 | 6840 | 4510 | 2790 | 4150 | 6000 | 3220 | 4030 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Average monthly discharges of the Danube River at the Smederevo hydrological station for the year 2024 according to the WMO program were notably high in January and October, and above normal ​​in February, June and September. In March and July, they were within the limits of normal values, and in April, May, August, November and December, within the limits of below normal values. The average annual discharge at the Smederevo hydrological station of the Danube River for the year 2024 was Qavg.annual =  5,270 m3/s.

Figure 6. Probabilities of exceedance of the average monthly discharges of the Danube River at the Smederevo hydrological station

Figure 6 shows the mean monthly discharges for the year 2024 together with the statistically calculated average monthly discharges of characteristic probabilities of occurrence at the Smederevo hydrological station on the Danube River.

According to the diagram in Figure 6, the average monthly discharges of the Danube River near Smederevo, during the year 2024, ranged from around values ​​corresponding to a exceedance of 5% to exceedance corresponding to values ​​of around 80%. From January (when the discharges were slightly above 5%) to May, they fell to a exceedance value of slightly below 80%. After a short-term increase in June, when they reached 30% exceedance, there was a discharge recession - in August, they fall to 80% exceedance. After a new increase during September and October, when the discharges again reached 5% of exceedance, from November they decrease to a value of 80% of exceedance, where they remain until the end of the year.

The confirmation of the previous findings can also be derived from the analysis of histograms of average monthly discharge volumes and corresponding double mass curves of monthly discharge volumes (Figure 7).

Figure 7. Histograms and double mass curves of monthly discharges volumes for the Danube River at the Smederevo hydrological station

Comparing the total water discharge of Danube River in the profile of the hydrological station Smederevo with the multiannual average, the monthly volumes of discharge water in the first two months of 2024 were above average, especially in January. In March they were around the average, but in April and May they were below the average. In June they are again above, and in July and August below the average. After a new increase in September and October, when the volumes of past water were higher than average, in November and December they were again below average values.

Generally, throughout 2024, the double mass discharge volume curve was slightly above the average multiannual curve, and the surplus created during the year resulted in an annual volume surplus in relation to the average was about 2310 × 106 m3. Based on that, when it comes to the Danube River near the hydrological station Smederevo, we can consider 2024 as an average water year.

3.	Tisa River

3.1        Senta hydrological station

Figure 8 shows the hydrograph of the Tisa River for 2024 at the Senta hydrological station with the corresponding minimum and maximum discharges envelopes and multiannual average.

Figure 8. Comparative hydrographs of extreme (min. & max.) and average multiannual discharges of the Tisa River at the Senta hydrological station

Based on the hydrographs shown in Figure 8, it can be concluded that the discharge values of the Tisa River ​​at the Senta hydrological station during January and February 2024 were above average. From March to September, a recession in discharge was observed, with occasional minor increases in June and July, but with values ​​lower than the multiannual average throughout the mentioned period. In August and September, the discharges approached the minimum envelope. From October to the end of the year, the discharges were on a smaller increase, but they were still below the multiannual averages. The minimum annual mean daily discharge for the year 2024 was recorded on September 12 and amounted to Qmin.year.avg. daily = 140 m3/s. The maximum annual mean daily discharge for the year 2024 was recorded on February 20 and amounted to Qmax.annual.avg.daily = 1,930 m3/s.

Table 3 presents the values ​​of mean monthly discharge values ​​at the Senta hydrological station on the Tisa River according to the WMO program for establishing a system for the standardized display of information on the current state of water resources and forecasting the hydrological situation for the next period (HydroSOS) for a given year in relation to historical multiannual average values ​​for a given month. Based on the data shown in Table 3, it can be concluded that the average monthly discharge values ​​in January and February were in the range of notably high values, in March around normal, while from April to July and from October to December they were below normal. In August and September, the average monthly discharges ​​were notably low. The average annual discharge of the Tisa River at the Senta hydrological station for 2024 was Qavg.annual = 692 m3/s.

Table 3:	Average monthly discharge values at the Senta hydrological station on the Tisa River according to WMO recommendations and their categorization (HydroSOS)

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qaverage |  | 711 | 822 | 1190 | 1420 | 1160 | 874 | 675 | 489 | 419 | 433 | 594 | 688 |
| Qmin |  | 134 | 182 | 233 | 402 | 308 | 259 | 140 | 127 | 125 | 118 | 182 | 183 |
| Percentiles (%) | 10 | 373 | 563 | 602 | 532 | 374 | 259 | 203 | 207 | 194 | 250 | 275 | 273 |
|  | 33 | 639 | 910 | 1030 | 805 | 579 | 435 | 314 | 265 | 292 | 344 | 421 | 418 |
|  | 50 | 749 | 1160 | 1270 | 1030 | 807 | 542 | 409 | 341 | 374 | 461 | 565 | 564 |
|  | 67 | 901 | 1370 | 1640 | 1360 | 962 | 734 | 548 | 468 | 457 | 593 | 762 | 742 |
|  | 90 | 1430 | 1820 | 2270 | 1920 | 1380 | 1290 | 898 | 748 | 777 | 1110 | 1320 | 1300 |
| Qmax |  | 2180 | 2300 | 2550 | 3130 | 3040 | 3190 | 2230 | 1990 | 1650 | 1240 | 2240 | 1890 |
| Qaver. mothly, 2024 |  | 1710 | 1670 | 1240 | 1000 | 578 | 507 | 399 | 193 | 184 | 261 | 275 | 299 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Figure 9. shows average monthly discharges for the year 2024, together with statistically computed average monthly discharges of characteristic probabilities of  occurrence at the profile of the Senta hydrological station on the Tisa River

Figure 9. Probabilities of exceedance of the mean monthly discharges of the Tisa River at the Senta hydrological station

From the diagram (Figure 9), it can be seen that the average monthly discharges in the profile of the hydrological station Senta na Tisa, in January and February 2024, moved slightly above 5% exceedance. In March they were between 30% and 50%, in April between 50% and 80%, and in the remaining part of the year, from May to the end of the year, the discharge values ​​ranged between 80% and 95% of exceedance.

An analysis of monthly discharge volumes using the histograms and double mass volume curves was also performed, as shown in Figure 10, was also conducted, confirming the previous conclusions.

Figure 10. Histograms and double mass curves of monthly discharge volumes of the Tisa River at the Senta hydrological station

When comparing the monthly discharge volumes of the Tisa River at the profile of the Senta hydrological station with the multiannual average, it can be concluded that in the first quarter they significantly exceeded the average, whereas in the remaining part of the year they were lower than the multiannual average. In summary, on an annual scale, the total discharge volume was lower than the multiannual average, resulting in an annual volumetric deficit relative to the average of 3,280 × 10⁶ m³.

Based on all of the above, when it comes to the Tisa near the hydrological station Senta, we can consider 2024 as a below-average (low-water) year.

4.	Sava River

4.1        Sremska Mitrovica hydrological station

Figure 11 shows the hydrograph of the Sava River for the year 2024 at the Sremska Mitrovica hydrological station, along with the corresponding minimum and maximum discharges and the multiannual average discharge.

Figure 11.  Comparative hydrographs of extreme (min. and max.) average multiannual discharges and discharges for the year 2024 of the Sava River at the Sremska Mitrovica hydrological station

As seen in Figure 11, in 2024, the Sava River discharges at the Sremska Mitrovica hydrological station were below the multiannual averages ​​for most of the year. The maximum annual average daily discharge was recorded on March 17 and reached the value Qmax.annual.avg.daily = 2250 m3/s. In the first quarter, the average daily discharges ​​were mostly around the multiannual averages. In the middle of March, there was a recession of the flow, so that in the first decade of May, they approached the minimum envelope. After stagnation during the rest of May, there is a small increase in June, but in July the discharges were again in recession and in August and the beginning of September the envelope again approached the minimum. The minimum annual  average daily discharge was recorded on September 9 and amounted to Qmin.year.avg daily = 307 m3/s. From the middle of September, there was an increase in discharge, so that in October they exceed the values ​​of average multiannual discharges. From the second half of October, there was a discharge recession until the third decade of November, when the discharges came closest to the envelope of minimum recorded multiannual discharges. In December, the discharges increase again, reaching the multiannual average.

Table 4 shows the comparative values ​​of average monthly Sava River discharges  at the Sremska Mitrovica hydrological station, according to the WMO program (HydroSOS).

Table 4: 	Average monthly discharge values at the Sremska Mitrovica hydrological station on the Sava River according to WMO recommendations and their categorization (HydroSOS)

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qaverage |  | 1810 | 1850 | 2180 | 2390 | 2040 | 1440 | 926 | 651 | 711 | 1050 | 1660 | 1940 |
| Qmin |  | 498 | 370 | 595 | 742 | 703 | 498 | 298 | 250 | 237 | 236 | 431 | 314 |
| Percentiles (%) | 10 | 948 | 1300 | 1370 | 1100 | 768 | 496 | 362 | 338 | 445 | 799 | 936 | 932 |
|  | 33 | 1550 | 1670 | 1940 | 1620 | 1120 | 708 | 464 | 491 | 668 | 1180 | 1440 | 1430 |
|  | 50 | 1860 | 2090 | 2370 | 1870 | 1320 | 863 | 569 | 605 | 879 | 1440 | 1890 | 1890 |
|  | 67 | 2140 | 2560 | 2630 | 2340 | 1650 | 1050 | 648 | 741 | 1210 | 1900 | 2240 | 2230 |
|  | 90 | 2690 | 3430 | 3730 | 3090 | 2180 | 1330 | 1020 | 1220 | 1790 | 2720 | 2920 | 2930 |
| Qmax |  | 3840 | 3790 | 4150 | 4930 | 4470 | 3120 | 3280 | 2330 | 3100 | 3660 | 3940 | 4180 |
| Qaver. mothly,2024 |  | 1780 | 1360 | 1610 | 1240 | 909 | 1100 | 537 | 389 | 645 | 1530 | 807 | 1530 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

From Table 4, it can be concluded that the average monthly discharge values ​​in January, September and December were in the range of normal values. In February, March, June, July, August and November the discharges were in the range of below normal, and in April and May in the range of notably low. Only in October the discharges were in the range of above normal values. The average annual discharge of the Sava River at the Sremska Mitrovica hydrological station for 2024 was Qavg.annual = 1120 m3/s.

The diagram (Figure 12) shows the average monthly discharges for the year 2024 together with the statistically calculated average monthly discharges of characteristic probabilities of occurrence at the Sremska Mitrovica hydrological station on the Sava River.

Figure 12. Probabilities of exceedance of the average monthly discharges of the Sava River at the Sremska Mitrovica hydrological station

From the diagram in Figure 12, it can be seen that in the first three quarters of 2024, the average monthly discharges were in the range of 50% to 95% exceedance. In October, the discharges increased to approximately 20% exceedance, and in November and December they were between 90% and 70% exceedance.

Histograms and double mass curves of monthly discharge volumes were prepared for the profile of the Sremska Mitrovica hydrological station on the Sava River (Figure 13).

Figure 13. Histograms and double mass curves of monthly discharges volumes of the Sava River at the Sremska Mitrovica hydrological station

From the graph in Figure 13, it can be seen that the volumes of average monthly discharges that passed through the profile of the hydrological station Sremska Mitrovica during almost the entire year 2024 were lower compared to the multiannual average, except in January when they were around the average and in October when they were above the average values. On the annual level, it can be concluded that the entire year was in volume deficit, by about 10,370 × 106 m3. Based on previous analyses, when it comes to the Sava River near Sremska Mitrovica, it can be considered that the year 2024 was a below-average water (low water) year.

5.	Velika Morava River

5.1        Ljubicevski Most hydrological station

Figure 14 presents the hydrographs of Velika Morava for 2024 at the Ljubicevski Most hydrological station, along with the corresponding envelopes and the multiannual average.

Figure 14. Comparative hydrographs of extreme (min. and max.), average multiannual discharges and discharges for the year 2024 of the Velika Morava River at the Ljubicevski Most hydrological station

During 2024, the Velika Morava at the Ljubicevski Most hydrological station did not have any significant episodes of increase and moved mainly between multiannual averages and envelope minimums, exceeding the average only in January and October. The maximum annual average daily discharge for the year 2024 was recorded on January 10 and amounted to Qmax.ann.avg.daily = 318 m3/s, and the minimum annual average daily discharge was recorded on September 9 and amounted to Qmin.ann.avg daily = 49.6 m3/s.

Table 5 provides the values ​​of average monthly Velika Morava discharges at the Ljubičevski Most hydrological station according to the WMO program (HydroSOS). From Table 5, it can be concluded that the average monthly discharges ​​in January and October were above normal, while in September and December they were in the range of normal values. In February, March, May, June, July, August and November, the average monthly discharges ​​were in the range of below normal, and in April in the range of notably low values. The average annual discharge of the Velika Morava River at the Ljubicevski Most hydrological station for the year 2024 was Qavg.annual= 150 m3/s.

Table 5: 	Average monthly discharge values at the  at the Ljubicevski Most hydrological station on the Velika Morava River according to the WMO recommendations and categorization (HydroSOS)

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qmean |  | 247 | 336 | 426 | 434 | 334 | 249 | 149 | 98.4 | 87.0 | 106 | 147 | 209 |
| Qmin |  | 53.4 | 81.9 | 126 | 114 | 76.0 | 63.0 | 38.5 | 29.3 | 27.2 | 29.0 | 52.0 | 47.4 |
| Percentiles (%) | 10 | 136 | 178 | 196 | 155 | 123 | 70.0 | 52.3 | 46.7 | 49.6 | 61.7 | 82.5 | 82.4 |
|  | 33 | 233 | 288 | 315 | 222 | 176 | 96.9 | 70.4 | 62.1 | 70.8 | 89.1 | 122 | 122 |
|  | 50 | 282 | 348 | 418 | 275 | 229 | 127 | 76.4 | 72.8 | 83.5 | 117 | 188 | 188 |
|  | 67 | 381 | 478 | 473 | 375 | 264 | 171 | 97.6 | 89.3 | 97.3 | 162 | 242 | 241 |
|  | 90 | 607 | 807 | 747 | 620 | 424 | 272 | 173 | 142 | 161 | 280 | 348 | 350 |
| Qmax |  | 638 | 1240 | 1050 | 1080 | 981 | 947 | 419 | 440 | 298 | 553 | 607 | 748 |
| Qmean mothly,2024 |  | 284 | 180 | 192 | 183 | 189 | 171 | 87.1 | 55.1 | 82.4 | 153 | 77.3 | 150 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Figure 15 shows average monthly discharges for the year 2024 together with the statistically calculated flows of characteristic probabilities of exceedance at the Ljubicevski Most hydrological station on the Velika Morava River.

Figure 15. Probabilities of exceedance of the average monthly discharges of the Velika Morava River at the Ljubicevski Most hydrological station

The diagram in Figure 15 shows that from the beginning of the year to August, the average monthly discharges ​​varied between 30% and 80% exceedance. In September and October, there was a smaller increase in water discharge, with values ​​within a range between 30% and 10% of exceedance. In November and December, the average monthly discharges ranged between 80% and 50% exceedance.

It is interesting to look at the corresponding histograms and double mass curves of discharge volumes in Figure 16.

Figure 16. Histograms and double mass curves of average monthly discharge volumes of the Velika Morava River at the Ljubicevski Most hydrological station

From the graph in Figure 16, it can be seen that the average monthly volumes of Velika Morava water that passed through the profile of the hydrological station Ljubicevski Most during almost the entire year 2024 were less compared to the multiannual average, except in January and October when they were slightly higher than the multiannual average and in September when they were around the average. On an annual level, the volume deficit of the past water in relation to the multiannual average was about 2,700 × 106 m3. Based on all the above, when it comes to the Velika Morava River on the hydrological station Ljubicevski Most, we can consider 2024 as a below-average water (low water) year.

6. 	Conclusion

By comparing the hydrographs registered at the hydrological stations Bezdan and Smederevo on the Danube, Senta on the Tisa, Sremska Mitrovica on the Sava and Ljubicevski Most on the Velika Morava (Figure 17), we can generally state that we had a discharge recession since the beginning of the year with two episodes of higher discharge increases in June and September, primarily on the Danube, and on the Sava in October. On the Tisa near Senta, the discharges were also in recession from the beginning of the year until August, and later turned to stagnation, and on the Velika Morava near the Ljubicevski Most, the discharges were stagnant, with smaller increases in June and October, which were more a consequence of the effects of the Danube slowing down. It should be noted that during the period of low water on the Velika Morava River, there is an influence of the backwater from the Danube River.

Figure 17. Comparative hydrographs of the Danube, Tisa, Sava and Velika Morava for the year 2024

The Danube River at the Bezdan hydrological station: January, February, June, September and October above average; March, April, May, July, November and December around average, only August was below average water. Overall, based on the presented indicators, the year 2024 was above average in the profile of the hydrological station Bezdan on the Danube River (compared to the multiannual average).

The Danube River at the Smederevo hydrological station: January, February, June, September and October were above average; March and July were average, and April, May, August, November and December were below average. Overall, the year 2024 in the profile of the hydrological station Smederevo on the Danube River was an average discharge year (compared to the multiannual average).

The Tisa River at the Senta hydrological station: January and February were above average, March was around the average, and from April to the end of the year and the water level was below the multiannual average. Overall, the year 2024 was below average in the profile of the hydrological station Senta on Tisa River (compared to the multiannual average).

The Sava River at the Sremska Mitrovica hydrological station: with the exception of January when the discharges were around the average, and October when they were above the average, the rest of the year the discharges were below, and in April and May significantly below the multiannual average. Overall, the year 2024 at the Sremska Mitrovica hydrological statiton on the Sava River was below average water (compared to the multiannual average).

The Velika Morava River at the Ljubicevski Most hydrological station: January and October were at the level of average values, the other months were below average, only April was significantly below average in relation to the corresponding average multiannual discharges. Overall, year 2024  at the Ljubicevski Most hydrological station on Velika Morava river was below average water (compared to the multiannual average).

It is interesting to note that during the year 2024, the maximum average daily discharges ever recorded for certain days appeared on the Danube: near Bezdan in January, September and October, as well as near Smederevo in January and October.

According to the discharge data provided in this analysis, it can be concluded that in 2024 the Danube was above average at Bezdan and average at Smederevo. On the Tisa near Senta, on the Sava near Sremska Mitrovica and near the Ljubicevski Most on Velika Morava, discharges were below average.