## Republic Hydrometeorological

## Service of Serbia

## Analysis of the Hydrological Conditions in the Main River Basins within the Territory of the Republic of Serbia for the Year 2025

## BELGRADE, SEPTEMBER 2026

Table of Contents

1.	Introduction.........................................................................................................................	  1

2.	Danube River....................................................................................................................	  2

2.1	Bezdan Hydrological Station …....................................................................................	  2

2.2	Smederevo Hydrological Station..................................................................................	  5

3.	Tisa River.......................................................................................................................	  8

3.1	Senta Hydrological Station...........................................................................................	  8

4.	Sava River......................................................................................................................	11

4.1	Sremska Mitrovica Hydrological Station.....................................................................	11

5.	Velika Morava River......................................................................................................	14

5.1	Ljubičevski Most Hydrological Station........................................................................	14

6.	Conclusion   .......................................................................................................................	16

List of Figures and Tables

Figure 1:	Graphic illustration of hydrological station locations………............................................. 1

Figure 2:	Comparative hydrographs of extreme (minimum and maximum), long-term average, and 2025 average daily discharges for the Danube River at the Bezdan hydrological station………......................................................................................................................  2

Table 1:	Average monthly discharge values at the Bezdan hydrological station on the Danube River compiled in accordiance with WMO recommendations and their HydroSOS categorization …........................................................................................................................................  3

Figure 3:  	Exceedance probabilities of monthly average discharges for the Danube River at the Bezdan hydrological station................................................................................................. 3

Figure 4: 	Histograms and cumulative curves of average monthly discharge volumes at the Bezdan hydrological station on the Danube River ........................................................................... 4

Figure 5:  	Comparative hydrographs of extreme (minimum and maximum), long-term average and 2025 average daily discharges for the Danube River at the Smederevo hydrological station..................................................................................................................................  5

Table 2:	Average monthly discharge values at the Smederevo hydrological station on the Danube River compiled in accordiance with WMO recommendations and their HydroSOS categorization ...................................................................................................................... 6

Figure 6: 	Exceedance probabilities of monthly average discharges for the Danube River at the Smederevo hydrological station..........................................................................................  6

Figure 7:	Histograms and cumulative curves of monthly average discharge volumes at the Smederevo hydrological station for the Danube River........................................................  7

Figure 8:	Comparative hydrographs of extreme (minimum and maximum), long-term average and 2025 average daily discharges for the Tisa River at the Senta hydrological station.................................................................................................................................. 8

Table 3:	Average monthly discharge values at the Senta hydrological station on the Tisa River compiled in accordiance with WMO recommendations and their HydroSOS categorization ............................................................................................................................................. 9

Figure 9:	Exceedance probabilities of monthly average discharges on the Tisa River at the Senta hydrological station.............................................................................................................  9

Figure 10: 	Histograms and cumulative curves of monthly average discharge volumes for the Tisa River at the Senta hydrological station............................................................................... 10

Figure 11:	Comparative hydrographs of extreme (minimum and maximum), long-term average and 2025 average daily discharges for the Sava River at the Sremska Mitrovica hydrological station................................................................................................................................ 11

Table 4:	Average monthly discharge values at the Sremska Mitrovica hydrological station on the Sava River compiled to accordiance with WMO recommendations and their HydroSOS categorization.....................................................................................................................11

Figure 12:	Exceedance probabilities of monthly average discharges for the Sava River at the Sremska Mitrovica hydrological station .......................................................................................... 12

Figure 13: 	Histograms and cumulative curves of monthly average discharge volumes at the Sremska Mitrovica hydrological station on the Sava River...............................................................13

Figure 14: 	Comparative hydrographs of extreme (minimum and maximum), long-term average and 2025 average daily discharges for the Velika Morava River at the Ljubičevski Most hydrological station........................................................................................................... 14

Table 5:	Average monthly discharge values at the Ljubičevski Most hydrological station on the Velika Morava River compiled to accordiance with WMO recommendations and their HydroSOS categorization ................................................................................................. 14

Figure 15: 	Exceedance probabilities of monthly average discharges for the Velika Morava River at the Ljubičevski Most hydrological station............................................................................... 15

Figure 16:	Histograms and cumulative curves of average monthly discharge volumes at the Ljubičevski Most hydrological station on the Velika Morava River.................................. 16

Figure 17:	Comparative hydrographs for the Danube, Tisa, Sava and Velika Morava rivers for  2025................................................................................................................................... 17

Introduction

The analysys of the 2025 hydrological condition incorporated data from key hydrological stations along the Danube, Tisa, Sava, and Velika Morava rivers. This selection was designed to provide a comprehensive overview of the hydrological status across all major rivers in Serbia, which receive their primary inflows from distinct geographical regions, including the Alps and Central Europe, the Carpathians, and the Balkan Peninsula.

Within the territory of the Republic of Serbia, this analysis utilized data from the Bezdan and Smederevo hydrological stations on the Danube River, Senta on the Tisa River, Sremska Mitrovica on the Sava River, and Ljubičevski Most on the Velika Morava (Figure 1). To evaluate the long-term hydrological context, historical discharge data spanning the entire monitoring period up to and including 2024 were used.

Figure 1:  Graphic illustration of hydrological station locations

2. Danube River

2.1 Bezdan Hydrological Station

Recorded discharges for the year 2025, together with the corresponding long-term characteristic (minimum, average, and maximum) discharges on the Danube River at the Bezdan hydrological station,  are presented in Figure 2.

Figure 2:  Comparative hydrographs of extreme (minimum and maximum), long-term   average and 2025 average daily discharges for the Danube River at the Bezdan hydrological station profile

Based on the Danube discharge data for 2025, it can be observed that the discharges were below average values almost throughout the entire year. One minor increase was recorded in August, when the maximum annual average daily discharge (Qmax.ann.avg.daily) was reached on August 2, amounting to 3,520 m3/s. The minimum value (Qmin.ann.avg.daily) was recorded on October 24 and amounted to 1,080 m3/s. The discharge values dropped below the minimum envelope on two occasions: from May 23 to 24 and from June 28 to 29.

In accordance with the World Meteorological Organization (WMO) program for establishing a system for the standardized display of water resource status information and hydrological forecasting (HydroSOS), a statistical analysis of the hydrological data was performed. The HydroSOS categorization defines five status levels based on specific percentile intervals: notably high, above normal, normal, below normal and notably low.

Below, for each analyzed hydrological station, a classification of average monthly discharge values is presented according to the HydroSOS system. The percentile thresholds specified in the tables for the given year are determined in relation to historical long-term average values ending with the previous calendar year for that specific month. Table 1 provides the average monthly discharge values at the Bezdan hydrological station profile on the Danube River according to the HydroSOS categorization.

Table 1:   Average monthly discharge values at the Bezdan hydrological station on the Danube River compiled in accordance to WMO recommendations and their HydroSOS categorization

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qavg |  | 2010 | 2140 | 2410 | 2750 | 2820 | 2920 | 2660 | 2290 | 1940 | 1690 | 1740 | 1890 |
| Qmin |  | 1010 | 1040 | 1190 | 1330 | 1590 | 1620 | 1190 | 1040 | 1030 | 987 | 901 | 939 |
| Percentiles (%) | 10 | 1370 | 1380 | 1620 | 1770 | 1960 | 2100 | 1720 | 1330 | 1280 | 1040 | 1180 | 1250 |
|  | 33 | 1680 | 1830 | 2090 | 2310 | 2400 | 2420 | 2210 | 1780 | 1610 | 1450 | 1390 | 1540 |
|  | 50 | 1840 | 2050 | 2330 | 2580 | 2770 | 2570 | 2540 | 2210 | 1830 | 1660 | 1580 | 1800 |
|  | 67 | 2070 | 2300 | 2580 | 3040 | 3050 | 2980 | 2770 | 2530 | 2050 | 1850 | 1850 | 1990 |
|  | 90 | 3000 | 2950 | 3280 | 4090 | 3600 | 4260 | 3780 | 3490 | 2840 | 2430 | 2470 | 2800 |
| Qmax |  | 3830 | 4260 | 4250 | 5900 | 5750 | 7570 | 6090 | 5530 | 3930 | 3150 | 3910 | 3960 |
| Qavg.month.2025 |  | 1720 | 1560 | 1320 | 1630 | 1450 | 1760 | 1690 | 2110 | 1440 | 1310 | 1490 | 1480 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

The average annual discharge on the Danube River at the Bezdan hydrological station profile for 2025 was Qavg.ann. = 1,580 m3/s. According to WMO recommendations, the average monthly discharge values at the Bezdan hydrological station profile on the Danube River were within normal limits in January, August, and November. They were below normal in February, September, October, and December and notably low from March to July.

An additional statistical analysis of the average monthly discharges was performed. Figure 3 illustrates the average monthly discharges for 2025, alongside the calculated average monthly discharges for characteristic exceedance probabilities at the Bezdan hydrological station profile on the Danube River.

Figure 3: Exceedance probabilities of average mothly discharges for the Danube River at the Bezdan hydrological station

The hydrograph indicates that the average monthly discharges at the Bezdan hydrological station profile in 2025 ranged from the 50% exceedance probability domain in August to 99% in March and June. In May, these values reached 99.5% exceedance probabilities. Remarkably, on June 28, an absolute minimum discharge for that specific month since the inception of measurements at this hydrological station was recorded, amounting to 1,310 m3/s.

An analysis of total runoff volumes was also conducted using average monthly volume  histograms and cumulative volume curves, which are illustrated in Figure 4.

Figure 4.	Histograms and cumulative curves of average monthly discharge volumes at the Bezdan hydrological station on the Danube River

Based on the available data for average monthly discharges in 2025 at the Bezdan hydrological station on the Danube River, and as illustrated in the histogram (Figure 4), it can be concluded that discharges throughout 2025 were significantly below long-term average values, except in August when they fluctuated around the average. In May, the discharge value dropped to half of the long-term average.

Overall, the cumulative curve of the average monthly discharge volumes remained below the long-term average throughout 2025, except in August when it aligned with the average curve. This progress of the hydrological condition resulted in a total annual water volume at the end of 2025 that was significantly lower than the volume corresponding to the long-term average, creating a volume deficit of approximately 21,800 x 106 m3. It must be emphasized that this volume deficit at the end of 2025 is the highest recorded since this annual analysis was first conducted in 2014. Based on the foregoing, the year 2025 can be classified as a hydrologically dry (low-flow) year for the Danube River at the Bezdan hydrological station.

2.2 	Smederevo Hydrological Station

For the section of the Danube River downstream from the confluence with the Sava and Tisa rivers, data from the Smederevo hydrological station were analyzed and are presented in the hydrograph in Figure 5.

Figure 5. Comparative hydrographs of extreme (minimum and maximum), long-term average, and 2025 average daily discharges for the Danube River at the Smederevo hydrological station

An analysis of the hydrograph in Figure 5 indicates that the discharge values of the Danube River at the Smederevo hydrological station — similar to those at the upstream Bezdan hydrological station were predominantly below the long-term average values. During 2025, four minor increases occured (in January, April, August, and December), with their peaks reaching the long-term average line. The maximum annual average daily discharge for 2025 was recorded on April 6, amounting to Qmax.ann.avg.daily = 7,650 m3/s. The minimum annual average daily discharge for 2025 was recorded on July 7, amounting to Qmin.ann.avg.daily = 1,910 m3/s. During the summer period, discharges dropped below the minimum envelope on two occasions (from June 25 to July 2, and from July 4 to 10), at which point historical minimum daily average discharge values for those specific calendar days were recorded for the entire observation period.

Table 2 presents the average monthly discharge values at the Smederevo hydrological station on the Danube River, compiled in accordance with the WMO HydroSOS program (Hydrological Status and Outlook System). The table details the classification for the specified year relative to the historical long-term average values for each respective month.

Table 2:	Average monthly discharge values at the Smederevo hydrological station on the Danube River compiled in accordance with WMO recommendations and their HydroSOS categorization

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qavg |  | 5180 | 5480 | 6340 | 7220 | 6650 | 5950 | 4920 | 4010 | 3610 | 3640 | 4320 | 4980 |
| Qmin |  | 2460 | 2480 | 3550 | 3460 | 3580 | 2680 | 2170 | 1820 | 1780 | 1840 | 2170 | 2030 |
| Percentiles (%) | 10 | 3380 | 3630 | 4350 | 4620 | 4430 | 3810 | 3290 | 2530 | 2410 | 2280 | 2840 | 3120 |
|  | 33 | 4180 | 5020 | 5380 | 5910 | 5850 | 5040 | 4090 | 3180 | 2950 | 2850 | 3490 | 4140 |
|  | 50 | 5130 | 5480 | 6120 | 7010 | 6490 | 5770 | 4670 | 3850 | 3340 | 3450 | 3880 | 4640 |
|  | 67 | 5810 | 5960 | 6910 | 8000 | 7050 | 6540 | 5460 | 4510 | 3880 | 3980 | 4550 | 5490 |
|  | 90 | 7210 | 7240 | 8670 | 10200 | 9340 | 7820 | 6840 | 5790 | 5090 | 5550 | 6490 | 7160 |
| Qmax |  | 8690 | 8850 | 9940 | 13100 | 12400 | 13000 | 10800 | 8030 | 7410 | 7290 | 9750 | 9240 |
| Qavg.month.2025 |  | 4460 | 3530 | 4400 | 5800 | 4250 | 3170 | 2460 | 2980 | 2390 | 2420 | 3360 | 3590 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

According to the WMO program, the average monthly discharges of the Danube River at the Smederevo hydrological station were within `normal` range only in January. They were `below normal` in March, April, August, and from October to December, and `notably low` in February and from May to July. The average annual discharge on the Danube River at the Smederevo hydrological station for  2025 amounted to Qavg.ann. = 3,570 m3/s.

Figure 6 shows the average monthly discharges for 2025 alongside the statistically calculated average monthly discharges for characteristic exceedance probabilities at the Smederevo hydrological station on the Danube River.

Figure 6: Probabilities of exceedance of the average monthly discharges for the Danube River at the Smederevo hydrological station

As illustrated in the diagram in Figure 6, the average monthly discharges of the Danube River near Smederevo during 2025 ranged a 70% exceedance probability in April to a 99% exceedance probability in June and July. Following January, when discharges stood slightly above the 80% exceedance level, a decline to nearly 95% exceedance was recorded in February. Subsequently, discharges increased through April, peaking at an exceedance level slightly below 70%. A recession period then ensued, causing discharges to drop to the 99% exceedance level in June and July. Following a minor recovery in August—when discharges reached the 80% exceedance level—a subsequent decline in September and October brought values close to 95% exceedance. After a brief resurgence later in October, discharges stabilized around the 80% exceedance level, where they remained until the end of the year. These findings are further corroborated by the analysis of the average monthly discharge volume histograms and the corresponding cumulative curves of monthly discharge volumes (Figure 7).

Figure 7. Histograms and cumulative curves of average monthly discharges volumes at the Smederevo hydrological station on the Danube River

When comparing the total discharge at the Smederevo hydrological station profile to the long-term historical average, monthly discharge volumes remained below average throughout 2025. This persistent trend resulted in an annual volume deficit of approximately 51,100 × 10⁶ m³. Notably, this deficit is the highest recorded at the end of a calendar year since this annual analysis was first conducted in 2014. Consequently, 2025 is classified as a hydrologically dry (low-flow) year for the Danube River at the Smederevo hydrological station profile.

3.	Tisa River

3.1        Senta hydrological station

Figure 8 illustrates hydrograph of the Tisa River for 2025 at the Senta hydrological station, alongside the corresponding minimum and maximum discharge envelopes and the long-term average.

Figure 8. Comparative hydrographs of extreme (minimum and maximum), long-term average, and 2025 average daily discharges for the Tisa River at the Senta hydrological station

Based on the hydrograph, it can be concluded that the Tisa River discharge values at the Senta hydrological station profile were below the long-term average during the first half of 2025. Following slight initial fluctuations, the first increase occurred in mid-March. After a recession lasting until mid-May, a subsequent increase briefly drove discharges slightly above the long-term average. This was followed by another recession until the beginning of July, when discharges fell below the minimum envelope. Discharges remained around the minimum envelope until November, when a new increase occurred, allowing values to reach the long-term average by early December. By the end of the year, discharges were again in decline, approaching the minimum envelope. The minimum average daily discharge for 2025 was recorded on July 9, amounting to Qmin.ann.avg.daily = 120 m3/s. During the year, discharges fell below the minimum envelope on several occasions (from February 26 to March 1, from March 8 to 15, from May 11 to 13, from June 29 to July 1, and on July 12, July 20, August 13, and September 4 and 13—for a total of 23 days). The maximum average daily discharge for 2025 was recorded on May 26, amounting to Qmax.ann.avg.daily = 1,050 m3/s.

Table 3 presents the average monthly discharge values at the Senta hydrological statione compiled in accordiance with the WMO Hydrological Status and Outlook System (HydroSOS) for the specified year relative to the historical long-term average values for each respective month. Based on these data, it can be concluded that the average monthly discharges fell within the `normal` range only in November and December. In January, May, and June, they were classified within the `low` range, while from February to April and from July to October, they remained within the `notably low` range. The average annual discharge of the Tisa River at the Senta hydrological station for 2025 was Qavg.ann. = 365 m3/s.

Table 3.  Average monthly discharge values at the Senta hydrological station on the Tisa River compiled in accordance with WMO recommendations and their HydroSOS categorization

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qavg |  | 722 | 831 | 1190 | 1410 | 1150 | 870 | 672 | 486 | 416 | 432 | 591 | 684 |
| Qmin |  | 134 | 182 | 233 | 402 | 308 | 259 | 140 | 127 | 125 | 118 | 182 | 183 |
| Percentiles (%) | 10 | 270 | 374 | 564 | 606 | 534 | 374 | 260 | 201 | 200 | 194 | 250 | 277 |
|  | 33 | 446 | 641 | 912 | 1030 | 799 | 573 | 431 | 311 | 263 | 291 | 340 | 418 |
|  | 50 | 612 | 750 | 1160 | 1270 | 1030 | 807 | 538 | 404 | 335 | 371 | 460 | 564 |
|  | 67 | 797 | 911 | 1370 | 1630 | 1360 | 961 | 710 | 548 | 467 | 456 | 590 | 752 |
|  | 90 | 1230 | 1500 | 1820 | 2270 | 1920 | 1380 | 1290 | 890 | 744 | 776 | 1100 | 1310 |
| Qmax |  | 2180 | 2300 | 2550 | 3130 | 3040 | 3190 | 2230 | 1990 | 1650 | 1240 | 2240 | 1890 |
| Qavg.month.2025 |  | 386 | 337 | 472 | 592 | 566 | 449 | 169 | 162 | 186 | 188 | 387 | 482 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Figure 9 illustrates the average monthly discharges for 2025, alongside statistically calculated historical monthly discharges for characteristic exceedance probabilities on the Tisa River at Senta hydrological station.

Figure 9. Exceedance probabilities of the average monthly discharges for the Tisa River at the Senta hydrological station profile

As illustrated in Figure 9, the average monthly discharges on the Tisa River at Senta hydrological station during 2025 ranged from approximately a 65% exceedance probability in December to 98% in July. In January, discharges stood at the 80% exceedance level, then fell to 95%, where they remained until April. After a minor increase in June, they reached the 80% exceedance level; however, during July and August, they hovered around 99%, and in September and October, they remained at approximately 95% exceedance. After another resurgence in mid-October, discharges increased to slightly above the 80% exceedance level in November and December.

An analysis of monthly discharge volumes using the histograms and cumulative curves, as shown in Figure 10, further corroborated the previous conclusions.

Figure 10. Histograms and cumulative curves of average monthly discharge volumes at the Senta  hydrological station on the Tisa River

When comparing the total discharge volumes of the Tisa River at the Senta hydrological station profile to the long-term average, monthly discharges volumes remained significantly lower than the historical average throughout 2025. This applies particularly to the spring and summer months, when values were half the long-term average. In July, the recorded volume was approximately one-quarter of the long-term average for that specific month. In summary, the total annual discharge volume was significantly lower than the long-term average, resulting in an annual volume deficit of 13,400 × 10⁶ m³. Similar to the trends observed at the previously analyzed hydrological stations on the Danube River, the volume deficit at the Senta hydrological station at the end of 2025 represents the highest recorded value since the inception of this analysis in 2014.

Based on these findings, 2025 can be classified as a hydrologically dry (low-flow) year for the Tisa River at the Senta hydrological station.

4.	Sava River

4.1        Sremska Mitrovica hydrological station

Figure 11 illustrates the hydrograph of the Sava River for 2025 at the Sremska Mitrovica hydrological station, alongside the corresponding minimum and maximum discharge envelopes and the long-term average discharge.

Figure 11. Comparative hydrographs of extreme (minimum and maximum), long-term average, and 2025 average daily discharges for the Sava River at the Sremska Mitrovica hydrological station

As illustrated in Figure 11, the Sava River discharge at the Sremska Mitrovica hydrological station profile remained below long-term average values for most of the year. After a slight initial increase in mid-January, followed by a recession until the end of February, discharges rose again in March, approaching the maximum envelope by the end of the month. The maximum annual average daily discharge was recorded on April 1, amounting to Qmax.ann.avg.daily = 4,390 m3/s. At the beginning of April, a subsequent discharge recession ensued, causing values to approach the minimum envelope by mid-June. After stabilizing slightly above the minimum envelope, discharges experienced a slight increase at the beginning of August, followed by a decline back toward the minimum envelope by the end of the month. During this period, the minimum annual average daily discharge was recorded on August 28, amounting to Qmin.ann.avg.daily = 331 m3/s. Beginning in early September, an upward trend in discharges allowed values to recover to long-term average levels by November. By the second half of November, a brief recession occurred before discharges began to increase once more during the final ten days of the month, temporarily exceeding the long-term average until early December. In December, discharges decreased again, approaching the minimum envelope.

Table 4 presents the average monthly discharge values on the Sava River at the Sremska Mitrovica hydrological station compiled in accordance with the WMO Hydrological Status and Outlook System (HydroSOS) for the specified year relative to the historical long-term average values.

Table 4. Average monthly discharge values at the Sremska Mitrovica hydrological station on the Sava River compiled in accordance with WMO recommendations and their HydroSOS categorization

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qavg |  | 1810 | 1840 | 2170 | 2380 | 2020 | 1430 | 922 | 648 | 710 | 1050 | 1650 | 1940 |
| Qmin |  | 498 | 370 | 595 | 742 | 703 | 498 | 298 | 250 | 237 | 236 | 431 | 314 |
| Percentiles (%) | 10 | 840 | 950 | 1300 | 1330 | 1090 | 770 | 496 | 362 | 340 | 446 | 801 | 940 |
|  | 33 | 1480 | 1530 | 1640 | 1930 | 1600 | 1120 | 704 | 463 | 491 | 672 | 1170 | 1440 |
|  | 50 | 1770 | 1840 | 2080 | 2360 | 1870 | 1310 | 856 | 569 | 605 | 890 | 1440 | 1890 |
|  | 67 | 2160 | 2130 | 2560 | 2630 | 2320 | 1640 | 1050 | 647 | 725 | 1210 | 1900 | 2240 |
|  | 90 | 2730 | 2690 | 3430 | 3730 | 3090 | 2180 | 1330 | 1010 | 1220 | 1780 | 2720 | 2910 |
| Qmax |  | 3840 | 3790 | 4150 | 4930 | 4470 | 3120 | 3280 | 2330 | 3100 | 3660 | 3940 | 4180 |
| Qavg.month.2025 |  | 2000 | 1180 | 2170 | 2890 | 1490 | 650 | 401 | 420 | 481 | 552 | 1110 | 1320 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Based on the data presented in Table 4, it can be concluded that the average monthly discharges in January and March fell within the `normal` range. In February, May, and from August to December, the discharges were classified within the `below normal` range, while in June and July, they remained in the `notably low` range. Only in April did discharges rise into the `above normal` range. The average annual discharge on the Sava River at the Sremska Mitrovica hydrological station for 2025 amounted to Qavg.ann. = 1,220 m3/s.

Figure 12 illustrates the average monthly discharges for 2025, alongside statistically calculated historical monthly discharges for characteristic exceedance probabilties on the Sava River at Sremska Mitrovica hydrological station.

Figure 12. Exceedance probabilities of the average monthly discharges for the Sava River at the Sremska Mitrovica hydrological station

As illustrated in Figure 12, in January, the average monthly discharges stood near the 35% exceedance probability level, before falling to the 80% level in February. Following an increase in discharges at the end of February, values reached the 50% exceedance level in March and rose to slightly above 30% in April. A subsequent discharge recession ensued, causing values to drop to a 95% exceedance probability in June and July. After a minor increase in August, discharges reached the 80% exceedance level, where they remained until the end of the year.

Histograms and cumulative curves of monthly discharge volumes were constructed for the Sava River at Sremska Mitrovica hydrological station profile (Figure 13).

Figure 13. Histograms and cumulative curves of averge monthly discharge volumes at the Sremska Mitrovica hydrological station on the Sava River

As illustrated in Figure 13, the average monthly discharge volumes on the Sava River at Sremska Mitrovica hydrological station were above the long-term average in January and April, below average in February, and around historical average values in March. From May to the end of the year, discharges volumes remained below the long-term historical average. On an annual scale, it can be concluded that 2025 experienced a volume deficit of 9,680 × 10⁶ m³ compared to the average. Based on these analyses, 2025 can be classified as a hydrologically dry (low-flow) year for the Sava River at Sremska Mitrovica hydrological station.

5.	Velika Morava River

5.1        Ljubicevski most hydrological station

Figure 14 presents the 2025 hydrographs for the Velika Morava River at the Ljubičevski Most hydrological station, along with the corresponding minimum and maximum discharge envelopes and the long-term average.

Figure 14. Comparative hydrographs of extreme (minimum and maximum), long-term average, and 2025 average daily discharges for the Velika Morava River at the Ljubičevski Most hydrological station

During 2025, the discharge on the Velika Morava River at the Ljubičevski Most hydrological station exhibited three periods of increase. The first occurred in January and second in April, when values slightly exceeded the long-term average, followed by a third peak in November and early December, when discharges approached the maximum envelope. Throught the remainder of the yaer, discharges fluctuated between the long-term average and the minimum envelope, exceeding the long-term average line only in August and October. The maximum annual average daily discharge for 2025 was recorded on December 1 and amounted to Qmax.ann.avg.daily = 623 m3/s, whereas the minimum annual average daily discharge was recorded on October 2 and amounted to Qmin.ann.avg.daily = 30.1 m3/s.

Table 5 presents the average monthly discharge values for the Velika Morava River at the Ljubičevski Most hydrological station, relative to histrorical long-term average according to the WMO Hydrological Status and Outlook System (HydroSOS) for the specified year. Based on the data presented in Table 5, the average monthly discharges in January, April and October fell within the `normal` range. In February, March, and August, discharges were classified within the `below normal` range, while from May to July, and in September, they remained within the `notably low` range. In November and December, the average monthly discharges escalated into the `above normal` range. The average annual discharge for the Velika Morava River at the Ljubičevski Most hydrological station for 2025 amounted to Qavg.ann. = 152 m3/s.

Table 5. Average monthly discharge values at the Ljubičevski Most hydrological station on the Velika Morava River compiled in accordance with WMO recommendations and their HydroSOS categorization

| Month |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qavg |  | 247 | 334 | 423 | 431 | 332 | 248 | 148 | 97.9 | 86.9 | 107 | 146 | 208 |
| Qmin |  | 53.4 | 81.9 | 126 | 114 | 76.0 | 63.0 | 38.5 | 29.3 | 27.2 | 29.0 | 52.0 | 47.4 |
| Percentiles (%) | 10 | 99.8 | 136 | 178 | 192 | 155 | 124 | 70.2 | 52.4 | 47.0 | 50.0 | 61.8 | 82.6 |
|  | 33 | 167 | 232 | 284 | 306 | 221 | 172 | 96.5 | 69.4 | 62.2 | 71.1 | 86.5 | 122 |
|  | 50 | 209 | 275 | 347 | 416 | 271 | 228 | 126 | 76.4 | 72.9 | 84.0 | 117 | 188 |
|  | 67 | 273 | 379 | 472 | 473 | 374 | 262 | 170 | 95.4 | 88.5 | 97.6 | 161 | 241 |
|  | 90 | 452 | 606 | 807 | 745 | 618 | 421 | 271 | 171 | 142 | 161 | 279 | 347 |
| Qmax |  | 638 | 1240 | 1050 | 1080 | 981 | 947 | 419 | 440 | 298 | 553 | 607 | 748 |
| Qavg,2025 |  | 199 | 148 | 187 | 323 | 138 | 75.3 | 46.6 | 58.1 | 44.9 | 91.5 | 245 | 268 |

| notably high | above normal | normal | below normal | notably low |
| --- | --- | --- | --- | --- |

Legend:

Figure 15 illustrates the average monthly discharges for 2025, alongside statistically calculated historical monthly discharges for characteristic exceedance probabilities for the Velika Morava River at the Ljubičevski Most hydrological station.

Figure 15: Exceedance probabilities of the average monthly discharges for the Velika Morava River at the Ljubicevski Most hydrological station

The diagram in Figure 15 indicates that, from the beginning of 2025 through May, the avearge monthly discharge values ​​varied between 50% and 95% exceedance levels. Subsequently, a discharge recession occured in June and July, with values falling to 99% exceedance probability. In August and September, discharges ranged from 95% to 80% exceedance. Folowing an increase in October, discharges reached the 50% exceedance level during that month, and further escalted to 10% in November. In December, the average monthly discharges declined to a value corresponding to 30%  exceedance.

These findings are further complemented by the corresponding histograms and cumulative curves of monthly discharge volumes presented in Figure 16.

Figure 16. Histograms and cumulative curves of average monthly discharge volumes at the Ljubičevski Most hydrological station on the Velika Morava River

As illustrated in Figure 16, the average monthly discharge volumes for the Velika Morava River at the Ljubičevski Most hydrological station remained below the historical average for the majority of 2025, except in November and December, when they surpassed the long-term average. On an annual scale, the discharge volume deficit relative to the long-term average was approximately 2,580 × 10⁶ m³. Based on these findings, 2025 can be classified as a hydrologically dry (low-flow) year for the Velika Morava River at the Ljubičevski Most hydrological station.

6. 	Conclusion

Comparing the hydrographs recorded at the Bezdan and Smederevo hydrological stations on the Danube, the Senta on the Tisa, the Sremska Mitrovica on the Sava, and the Ljubičevski Most on the Velika Morava (Figure 17), it can generally be stated that a discharge recession prevailed during most of the year. This downward trend was interrupted by four periods of increased discharge in January, April, August, and December. These increases were primarily observed on the Danube River at the Smederevo station, largely driven by three discharge crests propagating form the Sava River (with the exception of August, which resulted from an upstream discharge increase on the Danube River). Conversely, on the Tisa River at Senta, discharges remained stagnant from the beginning of the year, exhibiting only minor increases in March, May, and December. Similarly, discharges on the Velika Morava River at the Ljubičevski Most were characterized by stagnation, with minor increases registrated only in April and November.

Figure 17. Comparative hydrographs of the Danube, Tisa, Sava, and Velika Morava Rivers for 2025

At the Danube River Bezdan hydrological station, discharges in January and August hovered near historical average, whereas values in February, September, October, and December remained consistently below it. Furthermore, from March to July, discharges fell significantly below the long-term average. Overall, based on the presented indicators, 2025 is classified as a hydrologically dry (low-flow) year relative to the long-term historical average at this station.

Regarding the Danube River at the Smederevo hydrological station, discharges fell within the average range in January, but dropped below average in March, April, August, and from October to December. Notably, discharges in February and from May to July were significantly below the long-term average. Overall, 2025 was characterized as a hydrologically dry (low-flow) year at this specific hydrological station.

For the Tisa River at the Senta hydrological station, discharges in January, May, and June were below average, while values from February to April and from July to October fell significantly below long-term average. Conversely, discharges in November and December hovered near the historical average. Overall, 2025 was classified as a hydrologically dry (low-flow) year for the Tisa River at this station.

At the Sava River Sremska Mitrovica hydrological station, discharges in January and March hovered near the historical average, while April discharges surpassed the long-term average. In contrast, the remainder of the year (February, May, and from August to December) exhibited below average conditions, with June and July values falling significantly below the long-term average. Overall, 2025 represents a hydrologically dry (low-flow) year for the Sava River at Sremska Mitrovica station.

Finally, at the Velika Morava River Ljubičevski Most hydrological station, discharges in January and April aligned with the average level, whereas values in February, March, and August remained below average. Furthermore, from May to July and in October, discharges fell significantly below historical average levels. Conversely, discharges in November and December surpassed the long-term average. Overall, 2025 is classified as a hydrologically dry (low-flow) year for the Velika Morava River at the Ljubičevski Most station.

According to the discharge data provided in this analysis, it can be concluded that 2025 was a critically dry year across all observed hydrological stations. This finding is further corroborated by the fact that the cumulative discharge volume deficit at the Danube River (Bezdan and Smederevo and Tisa River (Senta), represents the highest recorded values since the inception of this analysis in 2014.